# STUDENTS  DO  NOT  OPEN  THIS  TEST  OR

# BEGIN  UNTIL  INSTRUCTED  TO  START

# 2024  Examination  for  the

# National FFA Agricultural  Technology and

# Mechanical  Systems

Career   Development   Event

Answer  Key

Do  Not  Distribute

Name ________________________________________

Print  Name  Legibly

READ  THE  FOLLOWING  INSTRUCTIONS:

Mark  all  answers  on  the  scan-sheet  using  a  pencil.

•  	You  have  one  hour  to  complete  this  exam.

•  	You  may  write  on  this  exam,  but  information  on  this  exam  is  not graded.   Blank  sheets  of  paper  are  provided  if  additional  calculation  space  or scratch paper is  needed.

•  	If  a  diagram,  picture,  or  table  is  needed  to  answer  a  question,  the  question  will  refer  to  the  appropriate  figure/page.

•  	Read  each  question  carefully  and  determine  the  single  correct  answer.

•  	If  a  mark  on  the  scan  sheet  needs  to  be  changed,  completely  erase  the  incorrect  answer  and  clearly  mark  the  appropriate  answer  on  the  answer  sheet.

•  	Each  student  needs  a  calculator  to  complete  this  examination,  but  calculators  may  not  be  shared  between  students.

•  	Formulas and conversion values are provided.  Do  not  round off intermediate answers when using  the  calculator  to  solve  these  problems.

Students are NOT allowed to use any type of electronic communication device, including but not limited to cellular telephones, pagers, two-way radios, or PDAs, during the CDE on Wednesday or Thursday.  If a student uses, handles, or accesses any type of electronic communication device, she or he may be disqualified.  If a personal emergency should arise during the CDE, students should contact a CDE official immediately for assistance.

Order, Point Assignment, and AFNR Alignment* for Exam Questions  (2 points each)

| 1. Machinery (math) PST.03.03 | 6. Environmental (knowledge) ESS.05.01 | 11. Structural (knowledge) PST.04.01.02.c. | 16. Compact Equipment (math) PST.03.01 | 21. Electrical (math) PST.03.02.01.c |
| --- | --- | --- | --- | --- |
| 2. Electrical (Knowledge) PST.05.02 | 7. Machinery (knowledge) PST.03.03 | 12. Environmental (math) ESS.03.02 | 17. Structural (math) PST.04.03 | 22. Compact Equipment (math) CS.03.02 |
| 3. Compact Equipment (Knowledge) PST.03.03.02.b. | 8. Electrical (math) PST.04.04 | 13. Machinery (math) PST.03.03 | 18. Environmental (knowledge) CS.04.01 | 23. Structural (math) PST.04.02. |
| 4. Structural ( math) PST.04.02.01.b. | 9. Compact Equipment (math) PST.03.01 | 14. Electrical (knowledge) PST.04.04 | 19. Machinery (knowledge) PST.01.03 | 24. Environmental (math) ESS.05.01 |
| 5. Environmental (math) NRS.03.02 | 10. Structural (knowledge) PST.04.02.02.c. | 15. Compact Equipment (knowledge) PST.03.01 | 20. Electrical (math) PST.05.02 | 25. Machinery (math)PST.02.02 |

*Competency numbers (PST.02.02.) accompanying each question coincides with numbered AFNR performance indicators listed in the ATMS Handbook.

This  exam  begins  on  the  back  of  this  sheet.

# 2024  Written  Examination  for  the

# National  FFA Agricultural  Technology & Mechanical  Systems

# Career  Development  Event

Mark  all  answers  on  the  scan  sheet  using  a  pencil.       Read  each  question  carefully  and  mark  the  single  correct  answer  on  the  scan  sheet.   Each  student  needs  a  calculator  to  complete  this  examination,  but  calculators  may  not  be  shared  between  students.  Information  written  on  this  exam  will  not  be  graded.

You  may  write  on  this  exam,  but  only  the  scan  sheet  is  graded.

1.    Machinery:  The John Deere 6M series 105HP utility tractor is equipped with an AutoPower/IVT transmission-hydraulic system that requires a fluid change at 1500 hours. The fluid capacity for the system is 60 liters. As a dealership manager, you have four tractors to service. Which purchase option is the least expensive with the least amount of fluid left over?       1 liter = 0.264172 gallon
One gallon = $26.00	Five Gallon Container = $123.00	Fifty-five Gallon Drum = $1,247

60 liters x 0.264172 gal = 15.85032 gallons

15.85032 gal x 4 tractors = 63.40128 gallons need

A.      (1) fifty-five-gallon barrel and (9) one-gallon containers

B.      (2) fifty-five-gallon barrel

C.      (1) fifty-five-gallon barrel, (1) five-gallon container, and (4) one-gallon containers

D.    (12) five-gallon containers and (1) one-gallon container

2.   	Electrical: A technician needs to install an electronic device to determine the volumetric output of a water pump operating a center pivot irrigation system. What device does the technician install?

pg. 560 ch. 21 Agricultural Technical Systems and Mechanics book

A.    temperature sensor

B.                flow sensor

C.          pressure sensor

D.  photoelectric sensor

3.	Compact Equipment: When using a bench grinder safely, what is the maximum tool rest spacing?

Pg. 203 chapter 8 AMTS book GW, tool rest must be adjusted to less than or equal to 1/8” to prevent the workpiece from becoming trapped between the stone and the rest.

A.   1/8”

B.   1/4”

C.   1/2”

D.   3/4”

4.   	Structural: A large pasture needs to be sectioned into four smaller equal-sized square fields to allow for rotational grazing. The dimensions of the pasture include a length of 4350 feet and a width of 3582 feet. Three strands of four-point barb wire will be installed. Wire costs $120 per roll. Each roll has 1,320 feet of wire. Tax is 6.5% of the purchase price. Assume that you will need an additional 10% wire for any errors. A single 16-foot gate will be installed for the main pasture perimeter and a 16-foot gate for each of the four sections to allow livestock to rotate. No wire is needed where gates are installed. What is the cost of the barbed wire, including tax, for the project?

Perimeter of Rectangle = 2(Length + Width)

Perimeter Large pasture = 2(4350ft + 3582ft) = 15,864 feet

Dividing the field will require approximately 4350ft for length and 3582ft for width

16ft *5 = 80 ft

15,864 + 4350 +3582 – (80ft) = 23716 * 3 = 71,148 ft of wire * 1.10 = 78,262.8 ft of wire needed

78,263 / 1320 = 59.29 rolls or 60 rolls * $120 = $7,200 * 1.065 = $7,668 for the wire with tax

A.        $ 4,474.96

B.        $ 5,493.00

C.        $ 6,524.00

D.        $ 7,668.00

5.    	Environmental: During the silking stage, a corn variety has an evapotranspiration rate of 0.32 inches per day. The sandy loam soil has an available water storage capacity of 2.1 inches. The farmer would like to know when to schedule the next irrigation. How long (in days) will it take before the crop exhibits stress due to insufficient moisture?

Day until crop stress = Water Storage Capacity ÷ Evapotranspiration rate

2.1 inches ÷ .32 in/day = 6.5625 or 6.5 days, page 638 Chapter 25 Agricultural mechanics and technology systems

A.         4.0 days

B.         6.5 days

C.         7.5 days

D.         8.0 days

6.     Environmental: Using the NFPA symbol to the right, what is the level of health hazard for the fertilizer?

page 36 Chapter 2 Agricultural technical systems and mechanics

A.          0

B.          1

C.          3

D.          4

You  may  write  on  this  exam,  but  only  the  scan  sheet  is  graded.

7.    Machinery: For a hydraulic system, what is the result when a constant fluid flow rate encounters a reduction in the diameter of a hose?

pg 944 chapter 37 hydraulic and pneumatic power Bernoulli’s principle Agricultural Mechanics and technology systems

A.                Increase in fluid pressure

B.               Decrease in fluid pressure

C.               Decrease in fluid velocity

D.                      Decrease in flow rate

8.    Electrical: If the electric company charges $0.86 per kilowatt-hour, what is the cost to operate an electric water heater to raise the water temperature of 500 gallons from 60°F to 180°F over a three-hour period? 
Power in kilowatts=Gallons of water x Temperature Rise in degrees fahrenheit325 x Time to heat in hours

Pg. 539 chapter 21 Agricultural technical systems and mechanics book.

(500*120) ÷ (325*3) = 61.53846154 Kilowatts * 3* $0.87 = $158.7692308

A.            $52.92

B.            $62.92

C.            $92.87

D.          $158.77

9.    Compact Equipment: You have been asked to fabricate an engine mounting bracket. What is the approximate tap-drill bit size to make  a 1/4-28 threaded hole?

Tap-drill bit size = major diameter – (1 ÷ number of threads per inch)

page 91 chapter 3 hand tools agricultural technical systems and mechanics

0.25” – (1 ÷ 28) = .2142857”

A.            0.123”

B.            0.214”

C.            0.234”

D.            0.250”

10.   Structural: What is the common name for the layout tool provided in the picture below?

A.                     Plumb Bob

B.                     Plate Level

C.                     Line Level

D.         Carpenter’s Level

pg 66 chapter 3 hand tools agricultural technical systems and mechanics

11.   Structural: What is the in-line device designed and constructed to provide a liquid seal that prevents the back passage of sewer gas into the adjoining spaces where it is installed?

pg 426 ch 17 agricultural technical systems and mechanics

A.    plumbing fixture trap

B.    sanitary drainage system

C.    sewer cleanout

D.    sewer vent pipe

You  may  write  on  this  exam,  but  only  the  scan  sheet  is  graded.

12.   Environmental: If a center pivot irrigation system is 1220 feet long (has a 2440-foot diameter), approximately how many acres can be irrigated under 260 degrees of the pivot's travel?

Area of a circle  =   (π)  (radius)2                       π  =  3.14                    diameter  =   2    radius                                                     1 acre  =  43,560 square feet              1 mile  =  5,280 feet            Complete circle = 360 degrees

Acres  =  260⁰/360⁰ x  3.14  x  (1220)2  x  1 ac / 43,560 ft2   =   77.48761351ac

A.   72.6  acres

B.   75.4  acres

C.   77.5  acres

D.   87.5  acres

13.   Machinery: Your current tractor produces 35 PTO-horsepower at a rated power take-off speed of 540 revolutions per minute. What horsepower is needed to operate a John Deere round baler that requires a PTO speed input of 540 rpm and a torque of 535 lb-ft? Does the 35 HP tractor have the capacity to operate the baler?

Torque  = PTO Horsepower x 5252 ÷ PTO RPM

PTO Horsepower = Torque x PTO RPM ÷ 5252

PTO HP = 535lb-ft x 540 ÷ 5252

PTO HP = 55.0076161

A.           25 HP, no

B.          35 HP, yes

C.          55 HP, yes

D.           55 HP, no

14.   Electrical: Use the motor wiring diagram provided. Which wires must be connected but isolated from the power supply line for 230 V operation with counterclockwise shaft rotation?

A.         T3, T2, T5

B.                T5, T4

Chapter 23 Ag Mech and Tech Systems, page 592

C.         T3, T2, T8

D.         T1, T3, T5

15.   Compact Equipment: What OSHA safety color designates machines or energized equipment that may cut, crush, or shock and emphasizes such hazards when a guard is removed?

Ch. 2 page 21 Ag Tech systems and mechanics.

A.             Red

B.        Orange

C.        Yellow

D.         Green

16.  Compact Equipment: A ride-behind gasoline mower must generate sufficient income from cutting grass to generate a profit over the purchase price, operating costs, and labor expenses.  A $1,200 mower operates 760 hours annually for three years and then sells at an estimated salvage value of $675.  If total labor and operating costs are $32 per hour and an average of 1850 square-feet of grass can be cut in an hour, what approximate amount should be charged for each hour of the mower’s operation to produce a 50% profit margin?

Straight Line Depreciation =  (Purchase Price – Salvage Value) ÷ (useful life of the equipment)

[ ($1800 - $675) / (3 yrs x 760 hrs/yr) ] + $32/hr   =  $32.49342105

$32.49342105 / 0.50  =  $64.98684211

A.          $ 55

B.          $ 60

C.          $ 65

D.          $ 75

17. Structural: A concrete retaining wall shown below is 30” thick.   Because of the sloping ground, the bottom of the concrete wall is angled, and the top is horizontal.  An additional 5% concrete is ordered to allow for measurement errors.  The Minimum concrete order is three yards, and concrete can only be ordered in half yard amounts above three yards. How much concrete will be needed to complete this pour?

1 cubic yard  =  27 cubic feet            1 cubic foot  =  1728 cubic inches               1 foot  =  12 inches

Volume  of rectangular prism =   (length)  x  (width)  x  (height) 		 = 3.14

Volume of triangular prism = 0.5 * b * h * length

A.         19.5 cubic yards

B.         20.5 cubic yards

C.         25.5 cubic yards

You  may  write  on  this  exam,  but  only  the  scan  sheet  is  graded.

D.         30.5 cubic yards

Divide the shape into a rectangular prism and a triangular prism

Convert the 30 inches to feet (30/12) = 2.5 ft width

Volume (cubic yards) of rectangular prism = (38*3*2.5) ÷ 27 = 10.55555556 cu. yds.

Volume (cubic yards) of triangular prism = (0.5*38*5*2.5) ÷ 27 =  8.796296296 cu. yds.

10.5555556 + 8.796296296 = 19.35185186 cubic yards * 1.05 for errors = 20.31944445 cubic yards

Figure 22-1 Chapter 22 sustainable energy systems Ag Tech Systems and mechanics book pg 568

18.  Environmental: Using 2017 preliminary data, what percentage of the U.S. Energy consumption relies on renewable sources?

A.      10 percent

B.      25 percent

C.      30 percent

D.      40 percent

19.   Machinery: For a 1.5” nominal pipe that is schedule 80, what is the inside diameter of the pipe?

Pg 949 ch 37 Ag Mech and tech systems book

A.       1.900”

B.       1.689”

C.       1.610”

D.       1.500”

20.   Electrical: A water pump for a stock tank has an electrical motor that operates at 120 volts and uses 5.8 amps of current.   If it is a 0.50 horsepower motor and operates with a power factor of 0.90, what is the approximate efficiency of the motor?      1 horsepower = 746 Watts

Horsepower  =     voltage  x  amperage  x  power factor  x  efficiency

746

0.5hp  =    120 V  x  5.8  A  x  0.90  x  Efficiency       >>>>>   Eff.  = .59546661558

746                                                       Eff.  =  59.5%

A.         59.5%

B.         73.5%

C.         83.5%

D.        88.5%

21.  	Electrical: The rooftop solar installation for a hog finishing barn has proven to reduce the monthly electric bill by an average of $80 for the four coldest months (winter) of the year, $145 for one month in the fall, $145 for one month in the spring, and $280 per month during the six warmer months of the year.  If the installation cost of the solar system was $45,000, excluding sales tax, approximately how many years must the system be utilized to achieve the economic “breakeven” point?

Breakeven in Years =  Installation Cost  /  ($ savings/Year)

Annual Energy Savings =  [ (4 mth x $80/mth) + (2 mth x $145/mth) + (6 mth x $280/mth)]   =    $2290/ yr

Breakeven = $45,000 / $2290/yr   =  19.65 yrs

A.      14 years

B.      15 years

C.      19 years

D.      20 years

You  may  write  on  this  exam,  but  only  the  scan  sheet  is  graded.

22.  Compact Equipment:  A partial picture of a 6-inch dial caliper is shown in the space below.  The jaws of the dial caliper are closed on a solid object and the measurement is displayed on the face of the dial caliper.  The measurement displayed on the dial caliper is between 3 inches and 4 inches.  What measurement is displayed on the dial caliper?

A.   3.365 inches

B.   3.465 inches

C.   4.365 inches

D.   4.465 inches

23.   Structural:  Building material costs often fluctuate.  The following table lists the prices for common size framing lumber in March 2020 and April 2024.  Which one of the lumber sizes listed in the table exhibits the highest percentage increase in cost?

| Lumber Size | Price in 2020 | Price in 2024 |
| --- | --- | --- |
| 2” x 4” x 8’ | $ 2.37 | $ 2.42 |
| 2” x 6” x 10’ | $ 5.37 | $ 18.98 |
| 2” x 8” x 10’ | $ 8.23 | $ 13.75 |
| 2” x 10” x 10’ | $ 10.81 | $ 18.98 |

$ 2.42 / $2.37 = 102.11%

$ 18.98 / $ 5.37 = 353.45%

$ 13.75 / $ 8.23 = 167.10%

$ 18.98 / $ 10.81 = 175.60%

A.  2” x 4” x 8’ lumber

B.  2” x 6” x 10’ lumber

C.  2” x 8” x 10’ lumber

D.  2” x 10” x 10’ lumber

24.  Environmental: A vertical distance of 400 ft is measured between the intake and discharge of a hydroelectric energy system. The water flow rate is approximately 900 gallons per hour. How many kilowatt-hours can be potentially generated in a 12-hour period?

60 gallons per hour = 1 gallon per minute		Head in Feet x flow rate (gpm) ÷ 12 = Kilowatts

400 feet of head x 15gpm ÷ 12 *12 = 6000 kilowatt-hours

A.            24,000 Kilowatt-hours

B.            18,000 Kilowatt-hours

C.              6,000 Kilowatt-hours

D.              2,400 Kilowatt-hours

25.    Machinery:  The following measurements were taken to determine the wheel slippage in a loaded tractor’s operation.  Each rear wheels have a diameter of 4.15 feet, the test distance is 100 feet long, and the loaded tractor’s rear wheels make 8.75 rotations while traveling 100 feet.  What is the approximate wheel slippage for this tractor during each rear wheel rotation?

Circumference of a circle  =  (2)  x  (π)  x  (radius)              π  =  3.14             diameter of circle  =  (2) x (radius)

Wheel circumference  =  [ 2 x 3.14  x (4.15’ ÷ 2) ]  =  13.031 feet

8.75 rotations x 13.031 feet = 114.02125 feet in 8.75 rotations

114.02125 ft (-) 100 ft ÷ 114.02125 = .1229704 * 100 = 12.29704%

This represents 14.02125 feet slippage over 100 feet or

14.02125 feet / 8.75 revolutions  =  1.602428571 feet / rotation

A.  1.28 feet/rotation,   1.73%

B.  1.37 feet/rotation,   4.14%

C.  1.48 feet/rotation,   6.68%

D.  1.60 feet/rotation, 12.30%