- What minute series is this topographic map?

  - 60 minute

  - 30 minute

  - 15 minute

  - 7.5 minute

- What is the contour interval of this topographic map?

  - 5 feet

  - 10 feet

  - 20 feet

  - 30 feet

- A map scale of 1:24000 means that 1 inch on this map represents how many inches of ground distance?

  - 24

  - 2,400

  - 24,000

  - 2,000

- According to the map symbol used, the body of water in Powell Hollow connected to Wolf Creek in Section 27, T10N, R1E is classified as a:

  - Disappearing Stream

  - Perennial Stream

  - Intermittent Stream

  - Perennial River

- Section 2, T10N, R1E is in ___________ County.

  - Morgan

  - Monroe

  - Hindustan

  - Brown

- Approximately what percentage of Section 21, T10N, R1E is covered in woodland?

  - 25%

  - 50%

  - 75%

  - 90%

- The map symbol below found in the S ½ of Section 23, T11N, R1E symbolizes what type of area?

  - Marsh or swamp

  - Submerged marsh or swamp

  - Wooded marsh or swamp

  - Submerged wooded marsh or swamp

- What type of roadway is Highway 37 as seen in Section 14, T10N, R1W?

  - State route, secondary hard surface

  - US route, secondary hard surface

  - US route, dependable hard surface

  - State route, dependable hard surface

- What type of terrain would you expect to encounter if you were walking south from East Fork Honey Creek in Section 14, T10N, R1E?

  - Open space with gradual change in elevation

  - Marsh or swamp with rapidly decreasing elevation

  - Woodlands with rapidly increasing elevation

  - Woodlands with gradual change in elevation

- How many acres are in a typical section of land?

  - 36

  - 100

  - 640

  - 1,000

- The symbol below located in the SW ¼ of Section 22, T10N, R1E symbolizes what type of building?

  - School

  - Ranger Station

  - Medical Center

  - House of Worship

- The longitudinal coordinates shown at the bottom right-hand corner of this map are 86° 22’ 30”. What unit does the number 30 represent?

  - Degrees

  - Minutes

  - Seconds

  - Hours

- What legal description correctly describes the location of Honey Creek School?

  - S ½ of Section 22, T10N, R1E

  - N ½ of Section 22, T10N, R1E

  - S ½ of Section 22, T11N, R1W

  - S ½ of Section 22, T10N, R1W

- What is the elevation of the fire tower in Section 4, T10N, R1E?

  - 900 feet

  - 950 feet

  - 960 feet

  - 968 feet

- What type of railway is the Illinois Central in Section 2, T9N, R1E?

  - Railroad tunnel

  - Standard gauge railroad, single track

  - Standard gauge railroad, double track

  - Narrow gauge railroad, single track

Use this image below to answer 16 and 17.

- What is the correct description of the shaded parcel, assuming the white square represents a typical section?

  - NW ¼ of SE ¼

  - NE ¼ of SW ¼

  - NW ¼ of SW ¼

  - NE ¼ of SE ¼

- How many acres is the shaded parcel, assuming the white square represents a typical section?

  - 160

  - 80

  - 40

  - 20

- How many sections are in a typical township?

  - 10

  - 18

  - 24

  - 36

- What year was this quadrangle mapped by the geological survey?

  - 1948

  - 1947

  - 1946

  - 1927

- What instrument can be used to measure the area of Bean Blossom Lake in Section 17, T10N, R1E?

  - Stereoscope

  - Planimeter

  - Field Microscope

  - Wedge Prism