| Name_____________________________________________________ |  |
| --- | --- |
| Ethanol Activity 4 – Combustion |  |

Purpose

Cleaner burning. Energy efficient. Environmentally friendly. E85. The labels and promotion of vehicles and other energy consuming products make many of these claims. But what do they mean? Is ethanol better for the environment than gasoline?

Most energy used in the United States consists of burning a combustible material and harnessing the heat, or energy, produced. The burning of combustible materials releases harmful gases and soot into the air. Typically, fuels producing more soot tend to be more harmful to the environment and to people’s health. Fuels producing more heat per unit produce more energy.

There are many methods to compare the efficiency and the environmental impact of fuels. To determine efficiency, the heat released during the combustion process may be measured. Environmental impacts can be measured by the particulate matter, or soot, released into the air and by the amount of carbon dioxide produced. Carbon dioxide is a greenhouse gas found in the lower atmosphere. The increase of greenhouse gasses in the atmosphere can cause global temperatures to rise.

A common method to determine the presence of particular substances is through the use of indicators. An indicator solution changes color in the presence of the material for which you are testing. Bromthymol blue (BTB) is an indicator that turns yellow in an acidic solution and blue in a basic solution. When carbon dioxide is suspended in water, it forms carbonic acid. BTB can be used to indicate the presence of carbon dioxide in water. Sodium hydroxide is a base used to neutralize acidity. The amount of sodium hydroxide required to neutralize the solution indicates the concentration of carbon dioxide in the solution.

When collecting data, scientists use visual indicators as well as measurements. Data collected using visual observations is called qualitative data while measured data is called quantitative data. What qualitative and quantitative data can you use to compare fuels?

Materials

| Per pair of students: LabQuest2 Temperature sensor Forceps 2 empty soup cans Lighter 125ml plastic bottle with lid 2 stirring rods 2 single hole stoppers Ring stand 10cm ring Utility clamp 100ml graduated cylinder 2 30ml graduated cups Beaker tongs Paper towels Water | Per group of four students: - 60ml burner with kerosene - 60ml burner with ethanol - Bromthymol blue (BTB) Sodium hydroxide (NaOH) |
| --- | --- |
|  | Per student: - Lab apron - Safety goggles - Pencil |

CAUTION – You will be burning combustible materials, refer to local safety lab procedures for burning items.

Procedure

In this laboratory, you and a partner will investigate the efficiency and environmental impacts of two combustible fuels – kerosene and ethanol. Then you will compare the amount of heat released from combustion, particulate matter produced, and carbon dioxide emitted for each fuel source by collecting qualitative and quantitative data.

Part One – Predictions

In complete sentences on Activity 4 Student Worksheet, write a prediction of what you believe will occur in terms of heat released, soot production, and carbon dioxide emissions for each fuel source.

Part Two – Preparation

- Pour 10ml of water into each of the two graduated 30ml cups.

- Add two drops of bromthymol blue (BTB) to each cup.

- Swirl each cup to mix the liquids thoroughly.

- Set cups aside until Part Four.

Part Three – Heat Released

- Obtain a burner from your teacher. You will conduct three different tests on one fuel source, then exchange burners with the other pair in your group and repeat the tests.

- Put on your safety goggles and tie back long hair.

- Prepare the LabQuest2.

Connect the temperature sensor to the LabQuest2 and turn the LabQuest2 on.

Choose New from the File menu. On the Meter screen, select Rate on the touch screen.

Change the data-collection rate to 0.2 samples/second. Set the data collection length to 240 seconds.

| - Set up the apparatus as demonstrated by your teacher. (See Figure 1.) Use the graduated cylinder to measure and pour 100ml of cold water into the can. Insert a stirring rod through the holes in the top of the can and hold it in place with two one-hole stoppers. Position the can 5cm (~2 inches) above the burner. Use a utility clamp to suspend the temperature sensor in the water. The sensor should not touch the bottom or side of the can. |  |
| --- | --- |
|  | Figure 1. Apparatus. |

- Start collecting data.

Use the lighter to ignite the wick. Position the burner directly below the center of the water-filled can. CAUTION: Always keep hair and clothing away from open flames.

Tap the green start arrow to begin collecting data.

A real-time graph of temperature vs. time will be displayed on the LabQuest2 screen during data collection.

Temperature readings (in °C) can also be monitored to the right of the graph.

- Stir the water slowly and continuously using a stirring rod until data collection stops.

- After data collection has stopped, analyze the graph to find maximum and minimum temperature of the water.

Choose Statistics from the Analyze menu.

Record the Maximum (final) and Minimum (starting) temperature values recorded during data collection in Table 2 of Activity 4 Student Worksheet.

Select OK.

- Store the data from the first run by selecting the File Cabinet icon.

Part Four – Carbon Dioxide Emissions

Pull the burner away from the ring stand.

Remove the cap from the plastic bottle.

Carefully place the opening of the bottle over the flame.

Hold it snugly against the metal casing of the burner until the flame is extinguished.

With the bottle still inverted, raise the bottle just enough to screw on the cap.

Turn the bottle right side up.

- Uncap the bottle and quickly pour one graduated cup of the water and BTB mixture into the bottle. Your goal is to let as little oxygen into the bottle as possible.

- Recap the bottle immediately.

- Shake the bottle so the solution mixes with the gas.

- Pour the solution from the bottle back into the graduated cup.

- Observe the contents and record in Table 1.

- Add NaOH one drop at a time to the solution while your partner swirls the solution until the blue color returns and is the same as the second cup.

- Record the number of drops of NaOH required to return the solution to the original color.

Part Five – Clean Up

- Empty the can of water and place upside down on a paper towel. Use the beaker tongs to move the can as the can and water may still be hot.

Dispose of the BTB solutions as instructed by your teacher.

Rinse the graduated cups.

Part Six – Comparison Fuel

- Exchange burners with the other pair in your group to test the fuel source you have not tested.

Repeat Part Two, Three, Four, and Five with the second fuel source and a new soup can.

Part Seven – Particulate Matter

- Locate your two soup cans.

- Compare the amount and color of particulate matter on the bottom of each can.

- Record your observations on Table 1 of Activity 4 Student Worksheet.

- Clean up as instructed by your teacher.

Part Eight – Compare Quantitative Data

Compare the two samples by viewing both sets of quantitative data on one graph.

After data collection is complete, select Run 2 on the touch screen and select All Runs. Both runs will now be displayed on the same graph.

Examine the data points along the displayed curve. To examine the data on the displayed graph, select any data point. As you move the examine line right or left, the temperature values of each data point are displayed to the right of the graph.

Use the displayed graph to fill in Table 2 and answer the analysis questions on the student worksheet.

Part Nine – Analysis

- Share your results with your classmates as instructed by your teacher.

Record the results of the other groups in Table 3 on Activity 4 Student Worksheet.

Determine the class average for temperature change and drops of NaOH for each fuel.

Write a lab report on your findings.

Conclusion

- Why do you think gasoline is used more than other substances for a common energy source?

- What are the advantages and disadvantages of burning biofuels versus burning gasoline?

Name_____________________________________________________

Activity 4 Student Worksheet

Table 1. Predictions

Kerosene prediction:

Kerosene is a petroleum-based fuel used as jet fuel and a heat source.

Ethanol prediction:

Ethanol is a biofuel produced from plant matter used as a gasoline alternative.

| Table 2. Data Collection |  |  |
| --- | --- | --- |
|  | Ethanol | Kerosene |
| Color of BTB solution with carbonic acid |  |  |
| Drops of NaOH required to neutralize |  |  |
| Particulate matter on can |  |  |
| Initial Temperature |  |  |
| Final Temperature |  |  |
| Change in Temperature |  |  |
| Notes |  |  |

Analysis Questions

Which substance produced the most amount of soot? Least?

Which substance produced the most amount of heat? Least?

| Table 3. Class Average |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Group | Ethanol |  |  | Kerosene |  |  |
|  | Δ in temp | NaOH | Particulate | Δ in temp | NaOH | Particulate |
| Average |  |  |  |  |  |  |