2024 National Agriculture Technology & Mechanical Systems Career Development Event

Environment and Natural Resources Skill

| Station Number | Participation Number | State | Score |
| --- | --- | --- | --- |

You have been hired to grade a pad. You and your crew have set up your measuring equipment and are preparing to set cut, fill, and grade stakes. The Points you are measuring represent the corners of that pad.

Take a back-site and a front-site reading from the Benchmark (BM) to Point A.  Using your measurements to answer the following question below.

BM= 679’

|  | Answer (record answer to the nearest hundredth of a foot – 0.00) | Possible Points | Score |
| --- | --- | --- | --- |
| Back Site |  | (1 pts) |  |
| Distance from BM to Instrument |  | (2 pts) |  |
| Height of Instrument |  | (2 pts) |  |
| Front Site |  | (1 pts) |  |
| Elevation at Point A |  | (1 pts) |  |
| Distance from Instrument to Point A |  | (2 pts) |  |
| Total Elevation Change from BM to Point A |  | (2 pts) |  |
| Distance from BM to Point A |  | (2 pts) |  |

- What is the current slope in percentage? ________________________ (2 pts)

- What is the current slope ratio to the nearest whole number (example 2:1) _________ (2pts)

- Please mark the cut or fill stake on the drawing below to achieve a 1.5% slope of this pad from BM to Point A. The stake will be installed at Point A.    (2 pts)

Four stakes are provided; please use those stakes to answer the following questions.

4) What is the elevation at the hub? _____________ (1 pts)

5) How many feet of material will need to be removed? ___________(1 pts)

6) How many feet of material must be added? _________________(1 pts)

7) What is the slope of this earthworks project? _________________(1 pts)

8) What station marker is the stake indicating? __________________(1 pts)

9) How many feet from the beginning marker is this station marker indicating? _____________ (1 pts)

| A | B | C | D |
| --- | --- | --- | --- |
| E | F | G | H |
| I | J | K | L |
| M | N | O | P |

According to the land description, the starting point of this property is a USGS benchmark described thusly:

USC&GS BM D 43, 1934 In Marion County, Indianapolis West Quad., in the SW ¼ of Sec. 18, T. 16 N., R. 4 E., 2nd P.M.; at Indianapolis; near the southwest corner of the Indiana State Fairgrounds; set vertically in the concrete water table at the east side of the old Boulevard station, about 56 yards north of the center of the intersection of the abandoned railroad and 38th Street, 7 yards west of where the west rail was, 1 foot above the sidewalk; a U.S. Coast and Geodetic Survey benchmark tablet, stamped "D 43 1934". 2nd Order Elevation 736.613 feet N.G.V.D. 1929

The grid block above is all of section 18 of Township 16 North and Range 4 East of the 2nd Principal Meridian of Indiana (this is the fairgrounds you are currently on) broken up into ¼ of a ¼.

10) How much land area (in acres) is represented by each lettered square? ___________ (1pts)

11) Please list the letters of the blocks where the benchmark described might be found using this description. ________________ (2pts)

You have been provided two maps: one is a historical map of the grounds from 1925, just before this description was first written; the other is a modern map. Use both maps to complete the following task.

12) Please place a single sticker on the modern map where you think the USGS benchmark in the description is located.  (2pts)